| Topic heading | Syllabus Ref | Idea cluster | Question 1 | Question 2 | Question 3 | Question 4 | Question 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Electricity | GCSEPE 1 | Electrical charge and current | Electrical current is a measure of what? - The amount of energy that is transferred every second in J/sec - The amount of charge passing every second in coulombs / sec - The amount of energy transferred to each coulomb of charge by a power supply in J / coulomb. - How much a component resists the flow of charge in ohms. | Current is measured in amps. 1 amp means what? - Each coulomb of charge contains 1 J of energy. - 1 J of energy is transferred to each coulomb every second. - 1 coulomb of charge flows past a point in the circuit every second. - 1 coulomb of charge flows past a point in the circuit every hour. | If a current of 2 amps flows for 1 minute the total charge flowed is: - 2 coulombs - 0.5 coulombs - 120 coulombs - 30 coulombs | Which of the following equations does not show the relationship between current, time and total charge flow? - Q = I / t - Q = I x t - t = I / Q - I = Q / t | Which statement about the current at positions 1, 2 and 3 in the circuit below is true? 1 2 3 - The current will be greatest at 1 and lowest at 2. - The current will be greatest at 2 and lowest at 1. - The current at 1 and 2 will be the same and lowest at 3. - The current will be the same at all three points. |
|  |  |  | B | C | C | A | D |
|  | GCSEPE 2 | Current, resistance and potential difference | In which situation will the current be greatest? - When the power supply has a high voltage and the resistance in the circuit is also high - When the power supply has a low potential difference and the resistance in the circuit is also high. - When the power supply has a high potential difference and the resistance in the circuit is low - When the power supply has a low voltage and the resistance in the circuit is high | Which equation correctly describes the relationship between potential difference, current and resistance? - I = V x R - R = V x I - R = V / ! - V = I x R | In a simple series circuit, the potential difference of the power supply is 6V; the total resistance in the circuit is 24 ohms. What current flows? - 0.25 A - 4 A - 144 A - 18 A | I V I V I V I V A B C D Which graph below describes a conductor that obeys Ohms Law? | In the circuit below the supply potential difference is 6 V. The lamp will blow if a current greater than 0.5 A flows but needs 0.1 A to flow for it to glow. What would be an appropriate resistor to add to the circuit? - 6 Ohms - 18 Ohms - 120 Ohms - 6000 Ohms |
|  |  |  | C | D | A | D | B |
|  | GCSEPE 3 | Resistors | Which graph below best describes how current varies with potential difference for an ohmic conductor? A B C D | Which graph below best describes how current varies with potential difference for a filament lamp? A B C D | Which graph below best describes how current varies with potential difference for a diode? A B C D | Which statement about light dependent resistors is accurate? - Its resistance drops as the light intensity increases. - The current passing through it drops as the light intensity increases. - Its resistance increases as the light intensity increases. - Its resistance drops as the light temperature increases. | Which statement about thermistors is accurate? - Its resistance drops as the temperature decreases. - The current passing through it drops as the temperature increases. - Its resistance increases as the temperature increases. - Its resistance drops as the temperature increases. |
|  |  |  | D | C | B | A |  |
|  | GCSEPE 4 | Series and parallel circuits | What is the total resistance in the circuit below? - 11 ohms - 3 ohms - 19 ohms - 0.5 ohms | The circuit below contains two identical resistors; a lamp and a 9 V power supply. The current is 0.5 A. If the resistance of the lamp is 2 ohms what is the resistance of each resistor? - 18 ohms - 9 ohms - 4.5 ohms - 2.25 ohms | What could the resistance either side of the cell in the circuit below? - 1 ohm - 2 ohms - 7 ohms - 4 ohms | Which sum correctly describes the current measured at ammeter A1? - A1 = A4 – A3 – A2 - A1 = A4 + A3 + A2 + A5 - A4 – A3 – A2 – A5 - A1 = A4 + A3 + A2 | A series circuit contains a power supply and three resistors. One of the resistors is removed. What will definitely happen to the current? - The current will drop by one third. - The current will increase by one third. - The current will decrease. - The current will increase. |
|  |  |  | C | B | A | D | D |
|  | GCSEPE 5 | Direct and alternating current | What type of current is produced when a circuit is connected to a cell? - Alternative current - Alternating current - Direct current - Direct and alternating current | Mains current has the following properties: - It has a voltage of 230V and a frequency of 230 Hz - It has a voltage of 50V and a frequency of 230 Hz - It has a voltage of 230V and a frequency of 50 Hz - It has a voltage of 50V and a frequency of 50Hz | Which graph of voltage against time best describes how the voltage changes with time in an alternating current circuit? V Time V Time V Time V Time A B C D | Which graph of voltage against time best describes how the voltage changes with time in an alternating current circuit? V Time V Time V Time V Time A B C D | What kind of current is generated at a power station? - Alternative current - Direct current - Direct and alternating current - Alternating current |
|  |  |  | C | C | B | A | D |
|  | GCSEPE 6 | Mains electricity, insulation, fuses and circuit breakers | Three core electrical cables use colours to show what the wires are for. The correct colours are: - Brown = live wire, blue = neutral wire and green and yellow stripes = earth wire. - Blue = live wire, brown = neutral wire and green and yellow stripes = earth wire. - Brown = live wire, green and yellow stripes= neutral wire and blue = earth wire. - Green and yellow stripes= live wire, blue = neutral wire and brown = earth wire. | What is the role of the earth wire? - To carry the alternating current from the supply. - To complete the circuit. - To act as a fuse - To stop the appliance becoming love. | The potential difference between the live wire and the earth is? - 240 V - 50 Hz - 230 V - 40 Hz | Sometimes appliances do not have earth connections. This is usually if? - They are made of metal - They are double insulated - They are broken - They use mains electricity | How are fuse wires are different from wires in the circuit? - Fuse wires are much stronger so they don't burn out. - They are thinner and burn out more easily than the wires in the circuit. - They are the same but aren't insulated. - Fuse wires only allow current to pass in one direction. |
|  |  |  | A | D | C | B | B |
|  | GCSEE 6 | Power | Which of the following equations shows an incorrect relationship between power, voltage and current - P = V x I - V = P / I - V = I / P - P = I x V | Which of the following equations describes how power, current and resistance are related? - P = I x R - P = I2 x R - P = I x R2 - P = I / R | 0.1 A passes through a device connected to the mains supply. What is the power of the device? - 23 W - 230 W - 2300 W - 0.0004 W | A 23 kW machine is connected to the mains. What current would flow? - 230 W - 529 W - 0.1 A - 100 A | Power is measured in watts. 1 W means? - 1 J of energy is transferred in total - 1 J of energy is transferred every second - 1 J of energy is transferred every minute - 1 j of energy is transferred for ever m of wire. |
|  |  |  | C | B | A | D | B |
|  | GCSEPE 7 | Energy transfer in everyday appliances | Which equation shows the total amount of energy transferred by a device of power rating P in time t? - E = P x t - E = P / t - E = t x P - E = P2 / t | A 1 kW hair dryer is used for 2 minutes. How much work was done? - 120 J - 120 kJ - 60 J - 60 kJ | In the equation E = Q x V, what does Q mean? - The current - The charge per second - The total number of coulombs. - The potential difference | A circuit was connected to a 9 V power supply and the total charge that flowed was 100 C. How much work was done? - 900 J - 900 W - 900 A - You can’t tell unless you know how long the current flowed for | In which situation was the energy transfer greatest? - 2 kW hairdryer on for 1 minute - A 30 W electric bulb on for 100 minutes. - A 10 kW fan heater for 10 seconds - A 10 W torch for 10000 seconds |
|  |  |  | A | B | C | A | B |
|  | GCSEPE 8 | The National Grid | Step up transformers do the following: - They increase V and I - They decrease V and I - They increase V and decrease I - They decrease V and increase I | Step-up transformers are used to increase the potential difference from the power station to the transmission cables because? - Power cables can’t manage low voltages - Too much energy is transferred from power cables at high currents. - Electricity travels faster at high voltages. - Low voltages cause problems for birds | What is the correct description of how the national grid uses transformers? - Step up upon going from power station to transmission cables and step down upon entering homes and factories. - Step up upon going from power station to transmission cables and step up upon entering homes and factories. - Step down upon going from power station to transmission cables and step down upon entering homes and factories. - Step down upon going from power station to transmission cables and step up upon entering homes and factories. | The system of cables and transformers linking power stations to consumers is called what? - Powrgen - The international Grid - National Power - The National Grid |  |
|  |  |  | C | B | A | D |  |
|  | GCSEPE 9 | Static Charge | What is transferred when an object gains a static charge? - Electrons - Voltage - Atoms - Protons | A spark can leap from an object with a static charge to another object if? - The other object is a metal - The other object is an insulator - If the potential difference between the two objects is high enough. - If a wire is placed between the two objects | Two identical insulators gained static charges when rubbed by identical clothes. The two objects were brought close to each other. What might be observed? - A spark would jump between the two objects. - The two objects would attract each other - The two objects would attract each other and a spark might jump between them. - The two objects would repel each other | Very different clothes rubbed two identical balloons. When brought close to each other the balloons were attracted to each other. Which statement is correct about the two balloons? - Both balloons have positive charges - Both balloons have negative charges - Both balloons transferred electrons to their cloths/ - Only one balloon transferred electrons to its cloth. |  |
|  |  |  | A | C | D | D |  |
|  | GCSEPE 9 | Electric fields | The area around a charged object where other charged objects will be attracted or repelled is called what? - An electric field - A magnetic field - An electric current - A magnet | The attractive and repulsive forces between electric fields are examples of what? - Contact forces - Magnetic forces - Non-contact forces - Gravitational forces | Which picture best describes the electric field around a positively charged ball? + - + - A B D C |  |  |
|  |  |  | A | C | A |  |  |